NEURO-FUZZY-INTEGRAL MODEL FOR HOLISTIC EVALUATION OF AN URBAN MAGNETIC LEVITATION TRANSPORTATION SYSTEM

Abstract

Nowadays, the management of industries or services must respect the principles of sustainability. The various aspects involved, inherent in the production of goods or facilities and in the environment, should be considered. An Environmental Management System (EMS) allows planning, organizing and carrying out the operational and environmental actions in a methodical and effective way. With reference to the implementation of an urban magnetic levitation transportation system, called MagLev-Cobra, this paper proposes a model of EMS, which results in the establishment of a Systemic Sustainability Indicator (SSI). This tool provides a simple but rigorous and objective method to analyze, compare and evaluate the impacts of the project under the principles of sustainability. Concepts of Fuzzy Theory and Neural Networks were applied to obtain the SSI, which brings together the 39 environmental indicators proposed in the EMS of the MagLev-Cobra system, allowing a holistic evaluation. Modeling adopted the standard ANFIS for grouping the indicators, full conversion of Fuzzy variables and indicators for DeFuzzyfication, has developed an application in Microsoft Access to generate the ISS. The model denoted that 69% of the indicators pointed adequacy MagLev-Cobra sustainability; 25% had adequate average, and only 12% characterized suitability to these precepts.

Keywords:	Magnetic	Levitation;	MagLev-Cobra;	Fuzzy;	Environmental Management System; Sustainability.

1. INTRODUCTION

Globalization, which is the result of the increase in commercial relations worldwide, brings together fast technological development determined by the necessity to reduce costs and to increase the level of the services provided, especially due to the consumer’s consciousness change regarding his role in the development of society.

In addition, there is an increasing need for using natural resources for society development, therefore creating a greater and more severe environmental impact and also leading to deterioration of the quality of life. The mentioned impact should be analyzed according to the socioeconomic, political and cultural contexts in which changes occur. From this point of view, it is important for the society to associate their development with environmental protection, that is, there should be sustainable development.

According to Newman et al. (1999) and  Ribeiro (2001), the concept of sustainability includes the greatest necessities these days, such as economic development, environmental protection (society depends on the environment to continue developing), and, finally, social justice. The world renowned definition for Sustainable Development, which has been widely used by laymen and experts from all over the world, was presented on the Brundtland report as “the development that fulfills the needs of the present without limiting the potential for meeting the needs of future generations” (CMAD, 1988).

Due to these aspects, it is worth noting that nowadays it is essential to manage productive systems, such as transport, or facility service systems which are appropriate to the principles of sustainability, considering the various aspects involved, such as those inherent in the production of goods and services and in the environment, which to a certain extent, is always in interaction, provoking and being provoked regarding its intrinsic features. More studies that link transportation and sustainability can be found in Litman (2008), MIT (2009) and EPA (2011).

2. OBJECTIVE

Based on an Environmental Management System (EMS), which was previously developed for operational control of the magnetic levitation transportation system called MagLev-Cobra, the objective of this paper is to suggest a model for obtaining a Systemic Sustainability Indicator (SSI) to analyze and represent the environmental conditions over time, according to the principles of sustainability.

The SSI will bring together the environmental indicators suggested by the EMS, allowing a holistic evaluation of the MagLev-Cobra system. In addition, according to management necessity, it will also be able to analyze the system under specific environmental concerns through partial indicators.

The Fuzzy Theory, together with the Neural Network techniques, will be used to create the SSI and the partial indicators. For its automation, the Excel and Access applications from Microsoft Office Professional will be used.

This paper is also aimed at applying the model to determine the partial indicators, as well as the SSI, by using computerized applications, thus making it possible to validate the model.

3. BACKGROUND

When public urban transportation is not appropriately planned, its activities create negative impacts  (Paes, 2006) such as traffic jams, car accidents, and high operational costs, from an economic perspective; problems related to mobility, health and quality of life, from a social perspective, and air, water and soil pollution, as well as noise and visual pollution, from a physical perspective.

The lack of planning or inefficient planning and control of this facility helps to create environmental liabilities, that is, components in the physical, biotic and man-made environments which are overly damaged. If these components are not systematically recovered, they can increase significantly; therefore jeopardizing the quality of life of the population and continuity of services.

It is worth mentioning that the Brazilian Environmental Crime Law, N° 9.605 (1998), imposes penalty and management sanctions against people or entities performing activities that are harmful to the environment, even if they are performed involuntarily and indirectly.

Therefore, although it is well-known that the operation of urban transportation systems that use magnetism as the main technological line does not involve greenhouse gas emissions and does not produce noise or vibrations, it is important to study the environmental aspects involved in it. In addition, it contributes to the rationalization of the existing urban infrastructure.

Regarding urban mobility, initial research indicate that the use of the MagLev-Cobra as the main transportation option to be integrated with the existing infrastructure will make significant advances related to the rationalization of the public urban transportation system and will also make it possible to create a peripheral network for the great transportation corridors, thus reducing the saturation effects of urban traffic.

The Magnetic Levitation Vehicle is an energetically efficient transportation system that does not produce noise, vibration or gas emission. In addition, its implementation is cheaper than the implementation of other similar rail systems.

The MagLev-Cobra was developed based on the characteristics of Superconducting Levitation, in order to profit from the existing urban infrastructure. Its guiding principle is the possibility of making curves of small radius and keeping the load uniformly distributed over the road.

The Superconducting Levitation (SML) seems promising for medium speed transportation applications (lower than 70km/h). It is based on the diamagnetic property of the superconductors, eliminating the magnetic field inside the superconductors.

The transportation capacity of each MagLev train increases as more units are added according to demand. The modules with the doors can be arranged in different ways throughout the vehicle, as each unity is an independent structure. The seats can be arranged transversally or vertically.

The MagLev-Cobra is driven by a short-primary type linear electric induction motor (LIM). It works with straight line motion rather than rotation and it is the component responsible for motorizing magnetic levitation trains, whose projects predict no contact with the guide way.

More information about MagLev-Cobra can be obtained in Stephan et al. (2007), Stephan et al. (2008) and David et al. (2009).

4. METHODOLOGY

In order to accomplish the proposed objective, the following steps were taken:

- 1st step: Characterization and adaptation of the Input Variables to convert them into Fuzzy variables. Analysis of the 39 environmental indicators of the EMS of the MagLev-Cobra system and the definition of coding, universe of discourse and its unit, linguistic terms and membership functions;

- 2nd step: Problem modeling in order to obtain the SSI. Use of Neural Network concepts and the ANFIS Model by grouping the input variables in a structured manner, thus making it possible to create the partial indicators that will converge into the SSI;

- 3rd step: Characterization and adaption of the Output Variables to transform them into Fuzzy Variables: by means of the network defined in the previous step, the output variables could be defined, as well as coding, the universe of discourse, the linguistic terms and membership functions;

- 4th step: Definition of the Fuzzy Integrals: in a Neural Network there are devices called neurons that process the input variables and transform them into output pulses. These devices, which are called multiplexers in this paper, have layers with specific activities. The first layer is called Fuzzification, as it receives the input variables and transforms them into Fuzzy sets via Fuzzy integrals;

- 5th step: Creation of rules of inference and generation of output pulses: the Fuzzy sets are processed by means of the rules of inference that will make it possible to generate output pulses through defuzzification. For this step it was necessary to develop a computerized application in Microsoft Access that creates the rules of inference, processes the results and generates the output pulses;

- 6th step: Model application: Using the computerized applications mentioned in the 4th and 5th steps, values will be submitted to the input variables of the model and they will generate the output pulses. These will be the input in the next multiplexers until the SSI can be obtained.

5. THEORETICAL BACKGROUND

5.1 THE FUZZY THEORY

The Fuzzy Theory emerged in 1965 with the publication of the paper entitled Fuzzy Sets on the Information and Control magazine, by Lofty A. Zadeh, from the University of California, Berkley. This Theory consists in modeling problems that can handle quantitative and/or qualitative variables by building algorithms based on the architecture of the consensual thought of experts or systems users. Zadeh et al. (1975) say that one of the great advantages of the Fuzzy Theory is to translate linguistic terms used in daily communication (natural language) into math expressions, by means of the properties of the Fuzzy sets.

In formal logic, a Fuzzy set “A”, defined in the universe of discourse “U”, is characterized by a membership function “µ A”, which maps the elements of “U” for the interval [0 ,1], i.e., µ A:U => [0,1]. A set of the classical set theory can be considered as a specific Fuzzy set which is usually called ‘crisp’, where µ A:U => {0,1}. Therefore, the membership function associates a real number “µ A(x)” to each element “x” of “U” in the interval [0,1], which represents the degree of possibility (or degree of membership) of the “x” element belonging to the set “A”.

The degree of membership µ A(xi) indicates the extent to which the element µ A(xi) belongs to the Fuzzy set A. Thus, this set can be represented by ordered pairs of a generic element “x” and their degree of membership of the A form = {x, µ A(x) | x ∈ U}. A Fuzzy singleton set “A” has a single point “x”, belonging to “X”, represented in the universe of discourse, with µ A(x) = 1.

In order to represent a discrete Fuzzy set A, the formation A= {µ A(x1) / x1, µ A(x2) / x2, ..., µ A(xn) / xn}is used, where the symbol "/" is only a separator. When the universe of discourse U

is discrete, the Fuzzy set A can be represented by A= ∑ µ A ( x) / x.

x∈U

When the universe of discourse U is continuous, the Fuzzy set A is expressed by the Fuzzy

integral as follows: A = ∫ µ A (x) / x.

U

It is worth mentioning that the symbols “∑” and “∫” are used to represent the logical union operation, thus they do not represent the summation and integration operators generally used.

5.2 NEURO-FUZZY NETWORKS

The Artificial Neural Networks (ANN) are distributed parallel systems that are inspired by the functional aspects of the human brain and which can learn to predict the real world and adapt to it. They consist of simple processing units, called “neurons”, which process certain relations. These units are arranged in one or more layers and are interconnected by connections.

The neuron model proposed by McCulloch and Pitts in 1943, which is called MCP, is a simplification of the biological neuron described at that time. It consists of n inputs and one output, as shown in Figure 1. (Medeiros et al., 2003).

Input 1

Input 2                                                     Output

Input n

Figure 1: MCP Neuron Model

According to Souza (1999), the Neuro Fuzzy Systems (NFS) are one of the most studied hybrid systems nowadays, as they associate the technical advantages of the ANN with the Fuzzy Theory. Internally, these systems map the Fuzzy regions from the input space in the Fuzzy regions of the output spaces by means of the Fuzzy inference rules.

The NFS consists of three stages of fuzzy reasoning called ‘layers’ (the name comes from the ANN), with the following features:

- Fuzzification layer of the input variables: it receives the input variables (crisp) and transforms them into Fuzzy sets;

- Intermediate layer (hidden layer): it receives the fuzzy sets from the previous layer and submits them to the fuzzy reasoning rules, thus producing output pulses;

- Defuzzification or output layer: it treats the output pulses and represents it as output variables, going back to the crisp set;

Sandri et al. (1999) and Ortega (2001) propose the representation of the input and output variables of the multiplexers (neurons) as X, U, T(X), M, where X is the name of the variable, U is the universe of discourse, T(X) is the set of names (linguistic terms) for the X values, and M is a function that associates a membership function to each element of T(X). Figure 2 shows an example of a graphical representation of a certain input variable.

Left

Center

Right

Unit



A=0

1

B0

D0

C0

Variable Name

E

Linguistic Terms

Universe of Discourse

Membership Degrees

Membership Functions

Figure 2: Example of a Graphical Representation of a Variable

One of the most known and applied neuro fuzzy networks is the ANFIS (Adaptive Neuro Fuzzy Inference System) structure proposed by Jang (1993). More details of this architecture may be found in Souza (1999), Pagliosa (2003), Ludwig Júnior (2004), Sandmann (2006), Becker Junior (2008) and Rodrigues (2010). This structure consists of 5 layers:

- Layer 1: it receives the input variables and fuzzy them;

  - Layer 2: each node of this layer represents a Fuzzy inference rule that defines the discharging degree of the rule when it receives the fuzzy input variables;

  - Layer 3: it is defined by the normalization of the discharging degree of each rule and it is considered to be a pre-processing of defuzzification;

  - Layer 4: we multiply the normalized discharging degree of the rule and the processed values in each rule, which correspond to the singletons or the linear combination of the input variables;

  - Layer 5: the output layer of each multiplexer is calculated, that is, the sum of the products of Layer 4 is defuzzified.

6. THE NEURO FUZZY INTEGRAL MODEL

Following the suggested methodology, Table 1 summarizes information about the input and output variables defined in steps 1 and 3. The following information is available for the variables: coding (the Ident. column), universe of discourse together with its unit (columns “U” and Unity, respectively), linguistic terms in column T(X) and membership functions, according to the pattern shown in Figure 2, which are characterized in the “Notable Values” column. These values apply to the Fuzzy integrals shown in the expressions 1, 2 and 3 defined in the 4th step. The variables are distinguished in the “Type” column, where “I” refers to Input and “O” refers to Output. A computerized application was developed on Microsoft Excel for the Fuzzy integrals processing and the input variable fuzzification.

| Left=0→130→11/X+30→150→0(x−C)(B−C)/X | (1) |
| --- | --- |
| Center=30→050→1(x−B)(C−B)/X+50→170→0(x−D)(C−D)/X | (2) |
| Right=50→070→1(x−C)(D−C)/X+70→1100→11/X | (3) |

Where B, C and D are notable values following the pattern of Figure 2, and x is an input value included in the universe of discourse X.

The modeling mentioned in step 2 of the methodology is shown in Figure 3, where the triangles represent the multiplexers. All the outputs of the triangles, except the SSI, represent the partial indicators of the model.

According to step 5, inference rules were created with the aid of the application developed by Microsoft Access, making it possible to obtain the partial indicators and the SSI.

With the exception of the SSI, the output variables of the multiplexers emit pulses that represent the partial inputs to those ahead.

Therefore, accomplishing the 6th step, the next topic will show the results due to the application of the Model.

ECI

Input 1

Input 2

Input 3

HR

Input 4

Input 5

Input 6

AC

Input 7

Input 8

Input 9

Input 10

Input 11

MN

Input 12

Input 13

Input 14

NR

Input 15

Input 16

MPI

Input 20

EN

Input 17

Input 8

Input 9

EA

NC

ST

TR

Input 25

Input 26

CF

Input 30

NRM

Input 31

Input 32

Input 33

QU

Input 36

RC

Input 37

Input 38

Input 39

OPI

SSI

SSI

Output 40

Output 41

Output 42

Output 43

Output 44

Output 45

Output 46

Output 47

Output 48

Output 49

Output 50

Output 51

Output 52

Output 53

Output 54

Output 55

Figure 3 – Multiplexer Aggregation for the ANFIS Neuro Fuzzy Network Formation

Table 1 – Input and Output Variables for the SSI Calculation

| Ident. | Name (X) | Type | Unity | Universe of Discourse (U) (U) | Linguistic Terms T(X) |  |  | Notable Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Left | Central | Right | A | B | C | D | E |
| ECI – Environmental Condition Indicator |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Nitrogen Dioxide Concentration in the Systems Region | I | µg/m3/ 1h | 0-3,000 | Low | Medium | High | 0 | 900 | 1,500 | 2,100 | 3,000 |
| 2 | Smoke Concentration in the Systems Region | I | 3 µg/m / 24h | 0-500 | Low | Medium | High | 0 | 150 | 250 | 350 | 500 |
| 3 | Carbon Monoxide Concentration in the Systems Region | I | ppm/ 8h | 0-40 | Low | Medium | High | 0 | 12 | 20 | 28 | 40 |
| MPI – Management Performance Indicator |  |  |  |  |  |  |  |  |  |  |  |  |
| 4 | Employees’ Satisfaction Funcionários | I | - | 0-10 | Not Satisfied | Satisfied | Very Satisfied | 0 | 3 | 5 | 7 | 10 |
| 5 | HR Qualification | I | - | 0-100% | Incapable | Being Qualified | Capable | 0 | 30 | 50 | 70 | 100 |
| 6 | Employees’ Schooling | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 7 | Evaluation of Accident Risks | I | - | 0-30 | Low | Medium | High | 0 | 9 | 15 | 21 | 30 |
| 8 | Workplace Accident Index | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 9 | Compliance with the Environmental Legislation | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 10 | Internal Audit Nonconformity | I | - | 0-10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |

Table 1 (cont.) Input and Output Variables for the SSI Calculation

| Ident. | Name (X) | Type | Unity | Universe of Discourse (U) (U) | Linguistic Terms T(X) |  |  | Notable Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Left | Central | Right | A | B | C | D | E |
| MPI – Management Performance Indicator |  |  |  |  |  |  |  |  |  |  |  |  |
| 11 | Legal Nonconformities Related to the Environment | I | - | 0-10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |
| 12 | Activity Efficiency of the Corrective Maintenance | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 13 | Activity Efficiency of the Preventive Maintenance | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 14 | Maintenance Program Quality | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 15 | Water Use | I | m3/day | 0 – 10.000 | Low | Medium | High | 0 | 3,000 | 5,000 | 7,000 | 10,000 |
| 16 | Energy Use | I | kW/day | 0 – 10.000 | Low | Medium | High | 0 | 3,000 | 5,000 | 7,000 | 10,000 |
| 17 | Environment Investment Index | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 18 | Environmental Suggestion Index | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 19 | Emergency Mocks | I | - | 0-5 | Low | Medium | High | 0 | 1.5 | 2.5 | 3.5 | 5 |
| OPI – Operational Performance Indicator |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Accessibility | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 21 | Safety | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 22 | Tariff | I | - | 0-100% | Low | Appropriate | High | 0 | 30 | 50 | 70 | 100 |
| 23 | Journey Time | I | - | 0 – 10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |

Table 1 (cont.) Input and Output Variables for the SSI Calculation

| Ident. | Name (X) | Type | Unity | Universe of Discourse (U) | Linguistic Terms T(X) |  |  | Notable Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Left | Central | Right | A | B | C | D | E |
| OPI – Operational Performance Indicator |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Noise Level during Operation | I | dB | 0-250 | Low | Medium | High | 0 | 75 | 125 | 175 | 250 |
| 25 | Quantity of Accidents | I | - | 0 – 10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |
| 26 | Acclimatization (vehicle and facilities) | I | - | 0 - 10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |
| 27 | Information for Users | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 28 | Systems Cleaning (vehicle and facilities) | I | - | 0 - 10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |
| 29 | Service Level1 | I | - | 0 -100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 30 | Average Occupied Area | I | Pax/m2 | 0 - 10 | Low | Comfortable | Tight | 0 | 3 | 5 | 7 | 10 |
| 31 | Hydric Efficiency | I | m3/Pax | 0 - 100 | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 32 | Energy Efficiency | I | kW/Pax | 0 - 100 | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 33 | Index of Complaints about the Environment | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 34 | Systems Quality | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 35 | The Clients’ Environmental Satisfaction Index | I | - | 0 - 10 | Low | Medium | High | 0 | 3 | 5 | 7 | 10 |
| 36 | Company x User Relationship | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 37 | Liquid Waste Recycling | I | - | 0-100% | Low | Medium | Appropriate | 0 | 30 | 50 | 70 | 100 |
| 38 | Solid Waste Recycling | I | - | 0-100% | Low | Medium | Appropriate | 0 | 30 | 50 | 70 | 100 |
| 39 | Innovation | I | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |

Table 1 – Input and Output Variables for the SSI Calculation

| Ident. | Name (X) | Type | Unity | Universe of Discourse (U) | Linguistic Terms T(X) |  |  | Notable Values |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Left | Central | Right | A | B | C | D | E |
| Output Variables |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 | ECI- Environmental Condition Indicator | O | - | 0-100% | Bad | Medium | Great | 0 | 30 | 50 | 70 | 100 |
| 41 | MPI – Management Performance Indicator | O | - | 0-100% | Bad | Medium | Great | 0 | 30 | 50 | 70 | 100 |
| 42 | OPI – Operational Performance Indicator | O | - | 0-100% | Bad | Medium | Great | 0 | 30 | 50 | 70 | 100 |
| 43 | HR – Human Resources | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 44 | AC – Work. Accid. | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 45 | NC – Nonconformity | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 46 | MN – Maintenance | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 47 | NR – Natural Resources | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 48 | EN – Environment | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 49 | EA – Environmental Appropriateness | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 50 | ST – Sustainability | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 51 | TR – Transportation | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 52 | CF – Comfort | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 53 | NRM – Natural Raw Materials | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 54 | QU – Quality according to the User’s Opinion | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 55 | RC – Recycling | O | - | 0-100% | Low | Medium | High | 0 | 30 | 50 | 70 | 100 |
| 56 | SSI | O | - | 0-100% | Inappropriate | Appropriate | Sustainable | 0 | 30 | 50 | 70 | 100 |

Key: I – Input Variable; O – Output Variable

7. THE APPLICATION OF THE NEURO FUZZY INTEGRAL MODEL - RESULTS

Table 2 shows the values of the input variables and Table 3 shows the results of the output variables after data processing by the computerized applications that are the result of the partial environmental indicators and the SSI.

Table 3 shows that the results can be either evaluated quantitatively, through the “Value” column, or qualitatively, through the “Dominant Qualification” column. In the latter, we can see green, yellow and red indicators, indicating the best condition, a warning situation and a non-satisfactory condition, respectively. The linguistic term follows what was shown in Table 1 and the percentage value next to it indicates how much the linguistic term is preponderant, where 100% reveals total representativeness.

Table 2 – Values of the Input Variables for the SSI Calculation

| Variable | Value | Variable | Value | Variable | Value | Variable | Value | Variable | Value |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1400 | 9 | 80 | 17 | 80 | 25 | 4 | 33 | 45 |
| 2 | 250 | 10 | 4 | 18 | 34 | 26 | 3 | 34 | 68 |
| 3 | 30 | 11 | 6 | 19 | 2 | 27 | 52 | 35 | 8 |
| 4 | 8 | 12 | 60 | 20 | 32 | 28 | 6 | 36 | 80 |
| 5 | 54 | 13 | 15 | 21 | 56 | 29 | 66 | 37 | 48 |
| 6 | 80 | 14 | 45 | 22 | 45 | 30 | 7 | 38 | 98 |
| 7 | 20 | 15 | 8500 | 23 | 4 | 31 | 72 | 39 | 35 |
| 8 | 80 | 16 | 4250 | 24 | 78 | 32 | 51 |  |  |

Table 3 – Results of the Output Variables for the SSI Calculation

| Variable | Value | Dominant Qualification | Variable | Value | Dominant Qualification |
| --- | --- | --- | --- | --- | --- |
| 40 | 64.33 | Great/72% | 49 | 90.00 | High/100% |
| 41 | 75.42 | Great/100% | 50 | 59.10 | Medium/55% |
| 42 | 53.27 | Average/84% | 51 | 29.36 | Low/100% |
| 43 | 80.86 | High/100% | 52 | 46.35 | Medium/82% |
| 44 | 89.04 | High/100% | 53 | 71.49 | High/100% |
| 45 | 70.00 | High/100% | 54 | 54.50 | Medium/77% |
| 46 | 33.49 | Low/83% | 55 | 72.22 | High/100% |
| 47 | 81.17 | High/100% | SSI | 77.44 | Sustainable/100% |
| 48 | 60.22 | High/51% |  |  |  |

From the results shown above, it is important to initially highlight the SSI result, which characterizes the MagLev-Cobra system as Sustainable. Table 4 presents the results of Table 3 in decreasing order, without the SSI value, so that the person who decides can evaluate which parts of the system managed by the EMS are the most critical ones, that is, which parts deserve more attention and which may require possible management attitudes, such as resource allocation, training needs etc. Concerning the “red” indicators shown above, we can see that they represent the MN-Maintenance and TR-Transportation parts, indicating that they are issues that are intertwined due to its objectives and that represent danger regarding environmental issues.

Table 4 – Results of the Output Variables in Decreasing Order of Value

| Variable | Value | Dominant Qualification | Variable | Value | Dominant Qualification |
| --- | --- | --- | --- | --- | --- |
| 49 | 90.00 | High/100% | 40 | 64.33 | Great/72% |
| 44 | 89.04 | High/100% | 48 | 60.22 | High/51% |
| 47 | 81.17 | High/100% | 50 | 59.10 | Medium/55% |
| 43 | 80.86 | High/100% | 54 | 54.50 | Medium/77% |
| 41 | 75.42 | Great/100% | 42 | 53.27 | Average/84% |
| 55 | 72.22 | High/100% | 52 | 46.35 | Medium/82% |
| 53 | 71.49 | High/100% | 46 | 33.49 | Low/83% |
| 45 | 70.00 | High/100% | 51 | 29.36 | Low/100% |

8- CONCLUSIONS

An EMS is oriented according to the management model based on a continuous cycle called PDCA, which sticks to the principle that everything should be planned before operating a system. In addition, after its effective operation, working conditions should be verified, operation should be reevaluated and confronted with what was previously planned, and feedback regarding process suitability, the scripts and working routine should be provided.

Focusing on MagLev-Cobra’s EMS indicators, which are essential to provide information necessary to decision-making, the use of the Neuro Fuzzy Integral Model – the focus of this paper – makes it possible to observe the evolution of the issues that maintain the system by means of the input variables, the partial indicators and the SSI.

The observation of the demand for certain parts of the system over time may indicate critical situations, optimizing the financial and human resources in search of sustainable transportation.

The use of the Fuzzy Theory and of the Neuro Fuzzy hybrid architecture made it possible to gather linear and nonlinear variables, with different dimensions, in only one model. In addition, it makes it possible to expand knowledge acquired by experts during the development of the MagLev-Cobra technology.

Concerning the necessity to develop computerized applications, it was observed that the architecture of the Neuro Fuzzy networks has made it possible to develop in a simple manner database to store the input variables and the results provided by the model, as well as the screens for its management. These can be achieved by using basic and low-cost applications if compared to Business Intelligence (BI) platforms and expert software.

9. REFERENCES

Becker Junior, Otávio Monteiro (2008) Criação do Pneumoperitônio por Punção no Hipocôndrio Esquerdo:	Sistema Nebuloso (Fuzzy) de Suporte de Decisão pela Relação Pressão/Volume do Posicionamento da Ponta da Agulha de Veress. Tese de Doutorado em Cirurgia e Experimentação. Universidade de São Paulo, São Paulo.

EPA – United States Environmental Protection Agency (2011) Sustainable Transportation Performance Measures, available http://www.epa.gov/smartgrowth/pdf/Sustainable_Transpo_Performance.pdf, Captured 08/02/2012.

CMAD - Comissão Mundial sobre Meio Ambiente e Desenvolvimento (1998) Nosso

Futuro Comum, Relatório Brundtland. Fundação Getúlio Vargas, Rio de Janeiro.

Jang, Jyh-Shing Roger (1993) ANFIS: Adaptive-Network-Based Fuzzy Inference System in IEEE Transactions on Systems. Man and Cybernetics. vol. 23, nº 3, pp.665-685.

Litman, Todd (2008) Sustainable Transportation Indicators - A Recommended Research Program For Developing Sustainable Transportation Indicators and Data, Sustainable Transportation Indicators Subcommittee of the Transportation Research Board (ADD40 [1]), available http://www.vtpi.org/sustain/sti.pdf Captured 08/02/2013.

Ludwig Júnior, Oswaldo, Schnitman, Leizer, Lepikson, Herman (2004) Uma Arquitetura Neural Híbrida para Extração de Regras Nebulosas. In IEEE Latin America Transactions vol. 2, nº 2.

Medeiros, Anderson V. de; Souza, Francisco Elvis C. e Maitelli, André L. (2003) Implementação de Um Sistema de Extração de Conhecimento de Redes Neurofuzzy. Universidade Federal do Rio Grande do Norte, II Workshop Técnico Científico do DIMAp.

MIT - Department Of Urban Studies + Planning (2009) Sustainable Transportation An International Perspective, MIT Journal Of Planning – Projections, volume 9.

Ortega, Neli Regina Siqueira (2001) Aplicação da Teoria de Conjuntos Fuzzy a   Problemas da Biomedicina. Tese de Doutorado. Universidade de São Paulo, Instituto de Física, São Paulo.

Paes, Gleicy Karen Abdon Alves. (2006) Sustentabilidade Ambiental dos Sistemas de Transportes Públicos em Centros Urbanos. Dissertação de Mestrado - IME, Rio de Janeiro.

Pagliosa, Angelo Luís (2003) Obtenção das Funções de Pertinência de um Sistema NeuroFuzzy Modificado pela Rede de Kohonen. Dissertação de Mestrado em Automação Industrial. Universidade do Estado de Santa Catarina, Joinville.

Ribeiro Suzana Kahn (2001) Transporte Sustentável – Alternativas para os Ônibus Urbanos. Centro Clima, COPPE/UFRJ, Rio de Janeiro.

Rodrigues, Marconi Câmara (2010) Identificação Fuzzy-Multimodelos para Sistemas não Lineares. Tese de Doutorado em Engenharia Elétrica e da Computação. Universidade Federal do Rio Grande do Norte, Natal.

Sandmann, Humberto Rodrigo (2006) Predição não-linear de Séries Temporais usando Sistemas de Arquitetura Neuro-Fuzzy. Dissertação de Mestrado em Engenharia, Escola Politécnica da Universidade de São Paulo, São Paulo.

Sandri, Sandra e Correa, Cláudio. (1999) Lógica Nebulosa. Instituto Tecnológico da Aeronáutica – ITA. V Escola de Redes Neurais, São José dos Campos: pp. C073-c090.

Souza, Flávio Joaquim. (1999) Modelos Neuro-Fuzzy Hierárquicos. Tese de Doutorado em Engenharia Elétrica. Pontifícia Universidade Católica do Rio de Janeiro – PUC-RIO, Rio de Janeiro.

Stephan, Richard M. and David, Eduardo G. (2007) Maglev-Cobra: O Transporte Urbano Sobre Trilhos Magnéticos, 3º Concurso de Monografia CBTU - A Cidade nos Trilhos, available http://www.cbtu.gov.br/monografia/2007/trabalhos/richardmagdalena.pdf, captured 08/02/2012, Rio de Janeiro.

Stephan, Richard M. and David, Eduardo G. (2008) Maglev-Cobra: Um Veículo Para Transporte Urbano Energeticamente Eficiente e Ambientalmente Correto, SOBRAEP – Sociedade Brasileira de Eletrônica de Potência, UFRJ – Universidade Federal do Rio de Janeiro, available http://www.sbpcnet.org.br/livro/60ra/textos/CO-RichardStephan.pdf, captured 08/02/2012, Campinas.

David, Eduardo G., João Fernando G. Tourinho and Stephan, Richard M. (2009) Sistema de transporte de levitação magnética supercondutora Maglev-Cobra, etapas para implantação

available http://www.brasilengenharia.com.br/ed/591/Eng_Transporte_II.pdf, captured 08/02/2012, Edição 591, pg. 143 a 146, Rio de Janeiro, 2009.

Zadeh, L. A. et al. (1995) Fuzzy Sets and their Applications to Cognitive and Decision Process. New York: Academic Press.

| Autor #1 |  |
| --- | --- |
| Name | Marcelo Prado Sucena |
| E-mail | marcelosucena@gmail.com |
| Function | Teacher |
| Institution | Federal University of Rio de Janeiro /COPPE |
| Departament | Electrical Engineering – Laboratory Applications of Superconductors - LASUP |
| City | Rio de Janeiro |
| State | Rio de Janeiro |
| Autor #2 |  |
| Nome | Richard Magdalena Stephan |
| Email | r.m.stephan@gmail.com |
| Function | Teacher |
| Institution | Federal University of Rio de Janeiro /COPPE |
| Departament | Electrical Engineering – Laboratory Applications of Superconductors - LASUP |
| City | Rio de Janeiro |
| State | Rio de Janeiro |
| Autor #3 |  |
| Nome | Vicente de Almeida Junior |
| Email | japeri@gmail.com |
| Function | Researcher |
| Institution | Secretary of Transportation of the State of Rio de Janeiro - SETRANS |
| Departament | Urban Transport Agency - AMTU |
| City | Rio de Janeiro |
| State | Rio de Janeiro |